# Enthusiast Lens

V1 Objective Vehicle Intelligence Schema & Evaluation Specification

Hackathon working specification - frozen design reference

V1 principle: The scored product output is objective and verifiable. Subjective judgments such as steering feel, exhaust sound quality, fun factor, ride quality, and interior quality are intentionally excluded.

# 1. Scope and Output Rules

The system resolves enthusiast-relevant facts for the exact vehicle configuration whenever evidence permits.

Configuration means, at minimum: model year, make, model, trim, transmission, drivetrain, market, and any package or option that materially changes a scored field.

The system may return Unknown when available evidence cannot support a confident answer. Unknown is preferable to invention.

Not Available means a fact is not reliably published or measurable for the case. Not Applicable means the field does not logically apply to that vehicle.

Derived values must be calculated deterministically from sourced inputs and identified as derived.

Every surfaced factual claim should retain provenance and a verification/confidence state.

# 2. Canonical Vehicle Context

## Vehicle Identity

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| model_year | integer | Exact model year. |
| make | string | Manufacturer/brand. |
| model | string | Exact model designation. |
| trim | string | Exact trim or subtrim. |
| market | enum/string | US market for the current hackathon evaluation set. |
| body_style | string | 2-door, sedan, SUV, etc., where needed to disambiguate. |
| transmission_config | string | Configuration selected on the listing, before detailed transmission fields are resolved. |
| drivetrain_config | string | RWD/FWD/AWD/4WD configuration selected on the listing. |
| package_options | array[string] | Known packages/options that affect enthusiast-relevant facts. |
| build_date_or_vin_evidence | string/null | Used only where within-model-year hardware changes require it, e.g. Tesla HW3/HW4. |

# 3. Objective Enthusiast Schema

## 3.1 Engine & Measured Performance

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| engine_displacement | number + unit | Factory specification. |
| engine_configuration | string | Cylinder count/layout and engine family where useful. |
| aspiration | enum | Naturally aspirated / turbocharged / supercharged / other. |
| horsepower | number + standard | Exact configuration output; do not inherit output from another application of the same engine family. |
| torque | number + unit | Exact configuration output. |
| curb_weight | number + unit | Exact configuration where available; transmission/drivetrain differences matter. |
| power_to_weight | derived number | Calculated deterministically from sourced horsepower and curb weight. |
| zero_to_60_mph | measured range/value | Instrumented test result(s), configuration matched. Preserve test provenance. |
| quarter_mile | measured range/value | Optional when reputable instrumented data exists. |
| skidpad_lateral_g | measured range/value | Instrumented lateral acceleration with test method/source retained. |
| braking_distance | measured range/value | Instrumented braking result, including speed interval/test method such as 70-0 mph. |
| exhaust_type | enum | Standard / factory performance / other objectively identified hardware. |
| active_exhaust_valves | boolean/unknown | Whether factory exhaust uses active/valved control. |
| selectable_exhaust_modes | boolean/unknown | Whether driver-selectable exhaust modes exist. |
| exhaust_outlet_configuration | string/null | Outlet count/layout when reliably documented. |
| exhaust_package_dependency | string/null | Trim/package/transmission dependency for performance or active exhaust hardware. |

## 3.2 Transmission

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| control_type | enum | Manual / automatic. |
| mechanism | enum | Traditional manual / torque-converter automatic / DCT / CVT-IVT / automated manual / other. |
| gear_count | integer/string | Number of fixed forward ratios; use appropriate representation for CVT. |
| gearbox_name_or_model | string/null | Specific transmission designation if reliably known. |
| paddle_shifters | boolean/unknown | Factory paddle controls. |
| manual_shift_mode | boolean/unknown | Driver-selectable manual ratio/step control. |

## 3.3 Drivetrain & Differentials

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| drive_layout | enum | FWD / RWD / AWD / 4WD. |
| awd_4wd_architecture | string | Full-time, on-demand, part-time, selectable; mechanical/electric where relevant. |
| center_coupling_or_transfer_case | string/null | Center differential, clutch coupling, transfer case, etc. |
| low_range | boolean/not_applicable | Presence of low-range gearing. |
| nominal_torque_split | string/null | Documented nominal distribution when meaningful. |
| variable_torque_behavior | string/null | Documented ability to redistribute torque. |
| front_differential_type | string/unknown | Open/LSD/electronic/locking/etc. |
| center_differential_type | string/not_applicable | Where applicable. |
| rear_differential_type | string/unknown | Open/LSD/electronic/locking/etc. |
| locking_differentials | array/string | Factory front/rear lockers and their control. |
| torque_vectoring | boolean/string | Presence/type of torque-vectoring system. |

## 3.4 Suspension, Axles & Chassis

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| front_suspension_architecture | string | MacPherson strut, double wishbone, multilink, etc. |
| rear_suspension_architecture | string | Independent/multilink/torsion beam/solid axle/etc. |
| front_axle_type | enum/string | Independent / solid / other where meaningful. |
| rear_axle_type | enum/string | Independent / solid / other where meaningful. |
| damper_type | string | Standard / sport / adaptive / electronically controlled, etc. |
| spring_type | string | Coil / leaf / air / other. |
| sway_bar_disconnect | boolean/not_applicable | Factory disconnect system. |
| chassis_or_strut_bracing | string/boolean | Factory bracing relevant to exact configuration. |

## 3.5 Brakes, Wheels & Tires

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| front_brake_type | string | Disc/vented/etc. |
| rear_brake_type | string | Disc/drum/vented/etc. |
| front_rotor_size | number + unit/unknown | Exact configuration if published. |
| rear_rotor_size | number + unit/unknown | Exact configuration if published. |
| caliper_type | string/unknown | Fixed/sliding. |
| caliper_piston_count | number/unknown | Front/rear as applicable. |
| performance_brake_package | boolean/string | Package/trim dependency and hardware change. |
| wheel_size | string | Diameter and width when known. |
| tire_size | string | Front/rear dimensions. |
| tire_category | enum/string | Summer / all-season / off-road / other. |
| staggered_or_square | enum/unknown | Staggered / square. |

## 3.6 Audio

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| audio_brand | string/unknown | Base or premium branded system. |
| speaker_count | integer/unknown | Exact factory count. |
| dedicated_tweeters | boolean/count/unknown | Presence/count if published. |
| subwoofer | boolean/count/unknown | Presence/count. |
| amplifier | boolean/string/unknown | Dedicated amplifier if documented. |
| system_wattage | number/unknown | Approximate factory-rated wattage when published. |
| audio_tier | enum/string | Base / premium / named system. |
| audio_package_dependency | string/null | Trim/package dependency. |

## 3.7 Driver Assistance & Highway Automation

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| adaptive_cruise_control | enum | None / basic ACC / full-speed / stop-and-go, based on actual capability. |
| acc_min_activation_speed | number + unit/unknown | Minimum operating/activation speed if documented. |
| acc_brakes_to_stop | boolean/unknown | Whether system can control to a complete stop. |
| acc_auto_resume | boolean/unknown | Whether it automatically resumes after a stop, subject to system rules. |
| lane_departure_warning | boolean/unknown | Warning-only capability. |
| active_lane_keep_assist | boolean/unknown | Active corrective steering assistance. |
| lane_centering | boolean/unknown | Continuous centering capability distinct from LKA. |
| hands_requirement | enum/unknown | Hands-on / hands-free / not applicable. |
| operating_domain | string/unknown | Compatible roads, mapped highways, speed/domain restrictions. |
| sae_level | enum/unknown | 0/1/2 where useful; not inferred from marketing names. |
| system_name | string/null | BlueCruise, Super Cruise, Autopilot, etc. |
| subscription_required | boolean/unknown | Whether continued capability requires subscription. |
| subscription_cost | currency + cadence/unknown | Current cost if relevant to the evaluated configuration/time. |
| trial_or_included_duration | string/null | Included period if applicable. |
| hardware_dependency | string/null | Required hardware generation/build-date dependency. |
| software_entitlement_dependency | string/null | Purchased/subscribed software dependency. |
| adas_package_dependency | string/null | Trim/package dependency. |

## 3.8 Configuration Dependencies (Cross-Category)

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| dependency_trigger | enum/string | Transmission / drivetrain / trim / package / build date / hardware generation / software entitlement. |
| affected_fields | array[string] | Fields changed by the dependency. |
| change_description | string | Objective description of hardware/capability gained, lost, or changed. |
| evidence | source reference(s) | Must support the exact dependency, not merely the broader model line. |

## 3.9 Provenance & Confidence (Cross-Category)

| Field / Subfield | Type / Values | Notes |
| --- | --- | --- |
| value_origin | enum | Car2DB structured / targeted web / manufacturer / instrumented test / derived. |
| source_name | string | Human-readable source. |
| source_url | string | Evidence URL when available. |
| source_type | enum | Manufacturer / technical documentation / instrumented publication / structured database / secondary source. |
| confidence | enum | Verified / supported / uncertain / conflicting. |
| conflicting_sources | array/object | Preserve material conflicts rather than silently choosing. |
| verification_state | enum | Verified / proposed / unresolved. |

# 4. Value-State Semantics

| State | Meaning | Scoring treatment |
| --- | --- | --- |
| Known value | Supported objective answer. | Scored normally. |
| Unknown | System cannot establish the fact with sufficient evidence. | Not a factual error, but reduces correct coverage when ground truth is knowable. |
| Not Available | Reliable ground truth is not available for that case. | Excluded from scoring for that field. |
| Not Applicable | Field does not logically apply to the vehicle. | Excluded from scoring unless the task explicitly evaluates N/A classification. |
| Conflict | Credible sources materially disagree. | Preserve the conflict; score using accepted ground-truth rules/ranges. |

# 5. Ground-Truth Rules

Ground truth must be independently sourced; Car2DB is not allowed to define the answer key because it is part of the Hybrid system being evaluated.

Factory equipment/mechanical specs: prefer manufacturer documentation, then authoritative technical documentation, then reputable automotive publications, then other secondary sources if necessary.

Instrumented performance: prefer reputable instrumented test publications. Manufacturer claims may be retained but must be identified as claims rather than independent measurements.

Exact factory facts use exact expected values when the exact configuration is known.

Instrumented numeric results that legitimately vary across comparable reputable tests use an accepted verified range rather than an average.

Do not widen accepted ranges automatically for obvious outliers or tests conducted under materially different configurations/conditions. Preserve those separately as evidence.

Every ground-truth fact records its source, source type, configuration match, and verification state.

Freeze the verified ground-truth fixture files before running comparative scoring. Neither evaluated agent receives access to them.

# 6. Evaluation Metric Rules

Primary metric: Correct Enthusiast Fact Coverage

Correctly surfaced scorable facts divided by all scorable ground-truth facts for the case.

Accuracy of attempted facts is reported separately so high precision from answering only easy fields cannot hide poor coverage.

Secondary metrics: factual error rate, Unknown rate, provenance/source success, web calls/searches per vehicle, latency per vehicle, and estimated model/API cost per vehicle.

A correct Unknown is preferable to an invented answer; however, Unknown reduces coverage when the fact was actually knowable.

Both Full-Web and Hybrid systems are run on the same 12 benchmark fixtures representing 11 vehicle families, with the same output schema, same ground truth, and same grader. The paired MINI fixture scores are averaged into one MINI family score, and headline Correct Enthusiast Fact Coverage is macro-averaged across the original 11 vehicle families.

# 7. Frozen Evaluation Set: 11 Vehicle Families, 12 Benchmark Fixtures

| # | Vehicle | Primary challenge |
| --- | --- | --- |
| 1 | 2026 Mazda MX-5 Miata Grand Touring Automatic | Transmission-dependent mechanical hardware/configuration changes. |
| 2 | 2020 MINI Cooper S 2-Door (ACC true-positive fixture) and 2021 MINI Cooper S 2-Door (ACC false-positive fixture) | Paired ACC true-positive/false-positive benchmark: vehicle-specific ACC verification and marketplace mislabeling. Two fixtures represent one MINI vehicle family for headline scoring. |
| 3 | 2022 Toyota GR86 Base | Trim-level equipment depth and option differentiation. |
| 4 | 2020 Ford Mustang EcoBoost Premium Fastback; 10-speed SelectShift automatic; 2.3L High Performance Package (67E); Ford Safe & Smart Package (77S); Equipment Group 201A; separately equipped B&O option 583 | Performance-package dependency; factory performance hardware/options. |
| 5 | 2024 Hyundai Elantra N Line | Same engine family with application-specific tune/output; N Line vs N contamination. |
| 6 | 2018 Cadillac ATS Base | Configuration ambiguity and wrong-spec inheritance across engine/drivetrain/trim. |
| 7 | 2025 Jeep Wrangler Rubicon 4xe | PHEV + 4WD/locker/axle architecture; ACC without assuming active lane-centering/LKA. |
| 8 | 2025 Dodge Charger Daytona | EV output ambiguity and trim/package-dependent performance hardware. |
| 9 | 2022 Kia Soul GT-Line Turbo | Near-identical trim/subtrim naming and correct turbo/DCT configuration resolution. |
| 10 | 2023 Tesla Model Y Long Range AWD | Mid-year HW3/HW4 transition plus software/hardware/entitlement-defined ADAS capability. |
| 11 | 2026 Subaru WRX Limited CVT | Transmission-dependent AWD architecture and system-depth resolution. |

# 8. Explicit V1 Exclusions

Steering feel or subjective steering quality.

Exhaust sound quality, loudness preference, or emotional character. Objective exhaust hardware remains in scope.

Fun-to-drive ratings.

Subjective handling impressions.

Ride quality judgments.

Interior/material quality judgments.

Reliability reputation.

Tuning potential and aftermarket support.

Subjective track ratings.

Scope freeze note: This document is the working V1 schema reference for the hackathon. Additions should be made only when a genuinely missing objective capability would materially affect the evaluation or the core enthusiast-shopping use case.